enable

conf t

hostname SW1

!

enable secret ifs

username admin privilege 15 secret KLNB2o

!

no ip http server

no ip http secure-server

no service telnet

ip domain name La_fibre_en_folie

crypto key generate rsa modulus 2048

ip ssh version 2

!

!

vlan 200

name Service_Logiciel

vlan 210

name Comptabilite

vlan 220

name Administratif

vlan 230

name Vente

vlan 240

name Supervision

vlan 250

name Administration_informatique

vlan 696

name Natif_securite

!

!

conf t

interface range FastEthernet0/3-7

switchport mode access

Switchport access vlan 200

end

conf t

interface range FastEthernet0/8-12

switchport mode access

Switchport access vlan 210

end

conf t

interface range FastEthernet0/13-17

switchport mode access

Switchport access vlan 220

end

conf t

interface range FastEthernet0/18-24

switchport mode access

Switchport access vlan 696

end

conf t

interface range FastEthernet0/1-2

switchport mode trunk

end

conf t

interface range GigabitEthernet0/1-2

switchport mode trunk

end

enable

conf t

interface range FastEthernet0/1-2

channel-protocol lacp

channel-group 2 mode active

end

enable

conf t

interface range GigabitEthernet0/1-2

channel-protocol lacp

channel-group 1 mode active

end

conf t

spanning-tree vlan 200-250,696 priority 61440

end