Ca marche

enable

conf t

hostname SW2

!

enable secret Ifs

username admin privilege 15 secret KLNB2o

!

no ip http server

no ip http secure-server

no service telnet

ip domain name La_fibre_en_folie

crypto key generate rsa modulus 2048

ip ssh version 2

!

!

vlan 200

name Service_Logiciel

vlan 210

name Comptabilite

vlan 220

name Administratif

vlan 230

name Vente

vlan 240

name Supervision

vlan 250

name Administration_informatique

vlan 696

name Natif_securite

!

!

conf t

interface range FastEthernet0/3-7

switchport mode access

Switchport access vlan 230

end

conf t

interface range FastEthernet0/8-12

switchport mode access

Switchport access vlan 240

end

conf t

interface range FastEthernet0/13-17

switchport mode access

Switchport access vlan 250

end

conf t

interface range FastEthernet0/18-24

switchport mode access

Switchport access vlan 696

end

conf t

interface range FastEthernet0/1-2

switchport mode trunk

end

conf t

interface range GigabitEthernet0/1-2

switchport mode trunk

end

enable

conf t

interface range FastEthernet0/1-2

channel-protocol lacp

channel-group 2 mode active

end

enable

conf t

interface range GigabitEthernet0/1-2

channel-protocol lacp

channel-group 1 mode active

end

conf t

spanning-tree vlan 200-250,696 priority 61440

---Garder en mémoire---

write memory

conf t

interface range FastEthernet0/1-3

switchport mode access

switchport access vlan 200

end

conf t

interface range FastEthernet0/4-6

switchport mode access

switchport access vlan 210

end

conf t

interface range FastEthernet0/7-9

switchport mode access

switchport access vlan 220

end

conf t

vlan 200

name Services_logiciels

end

conf t

vlan 210

name Comptabilité

end

conf t

vlan 220

name Administratif

end

Show run et swhow vlan pour vérifier

conf t

vlan 210

name Comptabilité

ip adress 192.168.3.253 255.255.255.0

end